June 2026 Work Post Fred Page Meeting that Identified data surface issues:

*** kind of want to add the condition stock data, SSB, for Inner bay of fundu versus SWNS to paint the picture of stable in scots bay and decline in German Bank, abundance at age between the spawning grounds

- Using castaway processed data

- 2009 data seems to have some surface trimming already, i.e. VIC2009057-009.

- Also some initialization issues? Or is this a feature in these areas? Artefact of taking CTD in waters that aren’t calm?

  - VIC2009057-060, 074, 075, 076, 077, 078, 079, 080, 081, 083, 084, 085, 086, 095, 096, 098

- Some historic data does not have a whole cast, just a surface and bottom temp. No salinity or density. Others have a few data points, i.e. at 0, 30, 53, 59, 63 and 111m. Other do have some salinity and density measurements. Others just salinity.

- Interesting – seems to be initialization issues in DFO data too:

  - 180377009-117555,122193

  - 180379012-121869

  - 180379016-121870

  - 180380009-122814

  - 180380018-141984

  - 180381015-122216

  - 180382003-119503

  - 180382016-121878

  - 180383021-146639

Some casts appear to have missing data:

18PE89001-154344, 154360

*** Near-surface CTD measurements (<5–7 m) show increased variability and non-monotonic structure consistent with sensor equilibration and platform motion, and were excluded from analysis.

Near surface contamination, dynamic error

- Bubble contamination: CTD spikes—especially in salinity—are widely attributed to air bubbles in the conductivity cell: Bubbles reduce conductivity → give downward salinity spikes. More common near the surface due to waves, ship motion, or partial out-of-water exposure [seabird.com]

- Mechanistically:

- Gas comes out of solution or gets entrained → attaches to sensors → intermittent noisy signal [link.springer.com]

- 👉 This lines up very well with your plots (spiky salinity and density near surface).

Because this surface pattern is consistent over years, instruments and personel, it effectively rules out instrument calibration issues, specific CTD model problems, data processing pipeline errors and operator specific handling. What remains is:

A fundamental property of CTD sampling in the near surface ocean

3 Key Issues QC

- Surface instability, resolve by trimming data at least top 5m

- Some casts have low resolution (prior to 2009)

- Incomplete variables, just temp or just salinity

Best overall approach

Build a common analysis product from both datasets:

- QC both sources consistently

- Put both onto the same depth support

  - either standard depth levels (e.g., 5, 10, 15, …, 35 m), or

  - fixed bins (e.g., 5–10, 10–15, …, 30–35 m)

- Reduce each cast to one value per depth level/bin

- Aggregate to a common time unit for trend analysis

  - usually yearly seasonal means, or

  - annual means if you have enough seasonal balance

- Then estimate trends over time, ideally separately by depth/bin and ground

That’s the most defensible way to answer a climate-change question from “historic DFO vs modern HSC”.

If there is no overlap (between HSC and DFO data, there isn’t), you should be careful to frame conclusions as:

- “apparent trend across combined record,” not

- “pure climate trend free of measurement effects”

# Statistical model I would use

## Best practical model

For each variable (temperature, salinity), fit something like:

value ~ year + source + ground + season + depth_bin

If you have repeated casts and uneven sample sizes, use a mixed model or at least aggregate first toYes — if your real question is climate change / long-term change, I would not make the main comparison “DFO vs HSC raw casts.” That would risk mixing up true temporal change with source / instrument / sampling-method differences.

The sources you found say two things that matter a lot here:

- Historical and modern profile datasets can differ because of instrument / sampling biases, and correcting those biases can materially change estimated warming trends. In one published comparison of historical bottle casts and CTD data, the authors found depth and temperature biases and showed that correcting them increased the estimated global warming trend for 1955–1990. [journals.ametsoc.org]

- For long-term climate work, profile data are commonly put on standard depth levels or otherwise harmonized vertically; NOAA’s World Ocean Database explicitly says it stores data both on the original observed depth levels and also interpolated to standard depth levels “to present a more uniform set of iso-surfaces for oceanographic and climate work.” [ncei.noaa.gov]

So the best approach is:

1) Put both sources onto a common vertical support

Because your HSC casts are much denser than your DFO casts, I would not thin HSC randomly.
Instead, I’d convert both to the same depth representation:

- either fixed bins (for example 5–10, 10–15, …, 30–35 m), or

- standard depths (for example 5, 10, 15, 20, 25, 30, 35 m)

That is the closest match to how long-term ocean climate products standardize profiles for comparison.

Suggested workflow

For each record (or cast/bin), keep:

- year

- ground

- JD_bin

- depth_bin

- Source

- variable of interest (Temperature, Salinity, Density, etc.)

Then compute annual seasonal means within:

- ground × JD_bin × depth_bin × year

This makes the trend analysis much cleaner.

So source differences and climate trends are confounded.
In that case, you can still analyze trends, but you need to be very cautious about attributing a shift entirely to climate versus methodology.

4) Fit a trend model that separates year, source, season, and region

value ~ year + Source + ground + JD_bin + depth_bin

Best practical pipeline for your case

Recommended main analysis

Depth-bin harmonization + annual seasonal trend model

Step 1: standardize profiles

For each cast:

- trim to 5–35 m

- bin to 5 m layers (or interpolate to standard depths)

Step 2: summarize per cast × depth bin

For example:

- cast mean temperature in each depth bin

Step 3: aggregate to annual seasonal means

Within:

- year × ground × JD_bin × depth_bin × Source

Step 4: examine overlap

If both HSC and DFO exist in the same years / bins / grounds, estimate source offset.

Step 5: fit trend model

Estimate whether temperature/salinity changed with year after accounting for source and seasonal/bin structure.

A very defensible “publication-style” framing

If I were setting this up, I’d state the analysis objective as:

“Estimate long-term changes in temperature/salinity in the 5–35 m layer while accounting for differences in vertical resolution and potential source effects between historical DFO profiles and modern HSC profiles.”

Bin both datasets to common depth intervals, compute season- and ground-specific annual means, and model trends over year with source included as a covariate (or calibration term if an overlap period exists).

If you want a simple first-pass analysis

Do this first:

- Bin by 5 m depth intervals

- For each cast/bin compute mean temperature

- Aggregate to year × source × ground × JD_bin × depth_bin

- Plot yearly means by source

- Check overlap years

- Then fit temperature ~ year + source + depth_bin + JD_bin + ground

That will tell you quickly whether the apparent change is mostly:

- a time trend

- a source difference

- or both